PNEUMATICS AND MECHATRONICS IN THE FOOD INDUSTRY

What the food industry asks for

The food industry represents a very broad sector, covering different and frequently automated stages of production, going from the procurement of raw materials to the market penetration, passing through the entire conversion process.

Machines and installations that use different technologies are implemented within the production cycle and are often attributed to pneumatics and mechatronics

The components employed in this field must display particular characteristics for specific usages, as well as high quality reliability standards.

For example, we refer to the fact that, when exposed to food, the components must not contaminate it or affect its taste. For this reason, it is necessary to use adequate and certified products for the specific usage in the food sector.

It is also important to take into account that machines and implants for food production are often washed or sanitised with particularly aggressive chemicals, reason why the components must be produced with materials resistant to chemical agents or, at least, protected from chemical assaults.

However, products that should not come into contact with food and shouldn’t be subject to chemical washes or assaults are also used in the food industry. These components, which are placed in different areas in comparison to the previous ones or simply protected by a dashboard or a panel, must often display characteristics such as flexibility, reconfigurability and speed.

Automation in the food sector and materials selection

Automation has become an essential point, with its own characteristics in each industrial sector. In particular, a very significant aspect in the food sector is the one concerning the employed materials, since the industrial components are constantly exposed to the products or belong to the equipment for its processing.

Metal alloys are very suitable materials, with particular reference to stainless steel. The most common ones are AISI 303 and AISI 304, because they are a good compromise between resistance to corrosion and workability. Then we go to AISI 316, which is more resistant to corrosion but with difficult workability, and to AISI 616L, where L means “low carbon”, useful to avoid the precipitation of chrome carbides, that means corrosion.

Brass is frequently used because it adds great ductility and workability to corrosion resistance, which means low-cost. Nevertheless, it is necessary to address the issue related to the release of metal components. On the market, there are several low-lead alloys, such as CW510L, that can be used immediately or after specific processes of extra deplumbing, in case of even more restrictive requirements, as in the case of the USA market.

Superficial treatments and superficial protections can be very useful to solve problems of food compatibility and/or resistance to corrosion.

Talking about pneumatic components in which the presence of sliding elements is constant, the selection of the gaskets materials is very important and for this reason, rubbers approved by FDA and other recognised bodies are increasingly used. Among the most common compounds in this field, there are FKM/FPM, EPDM and PTFE.

Finally, it is important to mention the lubricants, usually present in pneumatic or mechatronic components: here again, they have to be selected among the approved ones, usually by FDA.

Certifications

In the food sector, the legislation is wide and diversified according to the Nation where the machine or the system is produced. Indeed, there are different types of certifications. For example, in the United States the reference is NSF, explained through a series of rules (we recall NSF 61 for the products exposed to drinking water and NSF 169 for the products exposed to food). In such a case, the certification is voluntary and who wishes to work in the sectors covered by such rules, directly applies to the NSF body to obtain the worldwide-recognised certificates, after a long and expensive procedure.

Europe constitutes a different case, where regulations are more specific (and mandatory, since they have been accepted by the other Member States) and they define the operating procedures for the manufacturing of products that can be brought into contact with food. We refer to the European Regulation 1935/2004, related to the products, and to the Regulation 2003/94, related to good manufacturing practices (GMP). These regulations set out the guidelines that should be implemented according to operational guidance. However, at the moment Europe is not in possession of a communal operational line, reason why every Country acts independently or so. Therefore, there are different private bodies that release specific certifications, which are recognised in larger or smaller fields: like in the case of DVGW in Germany, WRAS in the United Kingdom, KIWA in the Netherlands and CSI in France.

How to move, then, in such a diverse scenery?

Fortunately, all these regulations have a common thread and are based on three different stages:

First of all, it is necessary for the materials to be chosen within approved lists (it is the case of FDA materials in the USA or materials belonging to the positive lists in Europe), which unfortunately do not include all types of materials.

Secondly, it is necessary to run specific release tests according to precise regulations or, without them, in accordance with more general guidelines.

Finally, it is necessary to be equipped with a system for the control of the process or, when required, to undergo regular audits of the certification bodies.

Most of the times, in order to obtain the certification it is necessary to address to an accredited laboratory, to which all the information about the product shall be provided: from the bill of materials to the list of materials and suppliers, up to the geometrical and functional characteristics of the product.

The laboratory will then make contact with the different producers of raw materials to verify their composition.

Therefore, it will run specific migration and release tests on samples taken from the production (type tests).

Finally, it will make sure to verify the production process with regular audits, in order to maintain its quality long-term.

In this filed, it is often necessary to ensure the production tracking along the entire chain that leads from the supplier of the components to the placing of the final product on the market. Indeed, in case of non-conformity, many regulations require the producer of the component intended to be exposed to food, to be able to perform recall campaigns of the defective products.

Components exposed to food or to zones subject to washings

Implementation is the basis of automation, and pneumatic cylinders are components, which are often exposed to food. Metal Work offers a wide range of stainless steel cylinders: cylinders under regulation ISO 15552 with diameters 16, 20 and 25 mm; round cylinders with diameters from 32 to 63. Where needed, polyurethane gaskets can be used, perfect for high speed and long live, or in KFM/FPM, for temperatures up to 150°C and/or resistance to corrosion.

To these cylinders, Metal Work has recently added a wide range of fittings and top-of-the-line accessories, manufactured in stainless steel AIDI 316L. Among the gripper quick fittings, we go from the straight fitting to the curve, from the rotary to the T type fitting. The available standard threads go from 1/8” to 3/8”, with pipe passage from ø4 to ø10. There is also a set of fittings with threaded mounts and one for the pipefitting. Among the most common accessories, we remind flux regulators and non-return valves.

Still on the food sector, Metal Work also offers the set of cylinders HCR (High Corrosion Resistance), which provides great levels of resistance to corrosion with a lower price in comparison to the stainless cylinders. This set, which has developed starting from a specific need of the dairy sector, is the result of numerous tests run with different materials and treatments, in cooperation with the University of Brescia. The behaviour is excellent in applications where there is exposure to aggressive environments (dairy, fruit and vegetables, food…) or to washings with harsh cleaners (lye, hydrochloric acid, lactic acid…). The HCR cylinders are compatible with a basic solution, pH of a maximum 12, acid solution with a pH of a minimum 2.5, in saline mist, according to the DIN 50021 for 500 hours. The white painting of the cylinder heads clearly distinguishes them from the standard cylinders.

Another fundamental component for the implementation through fluids is the fitting. As an alternative to the stainless steel fitting, Metal Work has recently reviewed its own set of brass fittings for the food sector. The revision has led to a set suitable for the European market, called F-E series, manufactured in deplumbed brass, and processed on its surface, with the usual bright grey colour. There is also a new set called F-NSF, adequate for both the European and the USA market, manufactured in low-lead brass with a further deplumbing treatment, without superficial processing, and distinguishable because of the yellow painting. In both series, gaskets in approved NSF Viton are used.

The anticorrosion Syntesi set is available for groups treating compressed air, filters, regulators, cut out switch valve etc. The metal parts are manufactured in processed brass or in stainless steel. For the rest, high-performance technopolymers are employed. Hence, this product is suitable for the employment in areas subject to water spray or not particularly aggressive cleaners.

Another issue concerns water-crossed pressure regulators for human consumption. They are employed in different cases, coffee machines, beverage distributors, dentist chairs. Metal Work offers the pressure miniregulators MRA BIT F series, in the versions for compressed air or water. The employed materials have been selected among the ones belonging to the lists approved at international level. It is a simple and economical item, which has already been produced in different configurations, in million pieces.

Even in the case of pneumatic valves, there are some stainless steel prototypes that are resistant to washings and corrosion. Furthermore, the most interesting solution is the one of the islands of solenoid valves, designed to be adequately inserted into a machine. Metal Work offers island valves, series HDM or EB80, for “splash area”. The concept they are built on is to split the system in two parts: the area with fittings for air pipes in the part of the machine subject to washings; the area with electrical connections and proper valves external to this zone, which are exempted from the necessity of being resistant to water or other washing fluids. A technopolymer flange, aluminium processed or in stainless steel (depending on the case), keeps the two areas separate by means of a gasket. The result is the advantage to freely wash the internal zone of the machine without exposing the electrical parts of the solenoid valves to damages or infiltrations.

Non-food contact components

We are now seeing the requirements for those components that are not in contact with food or subject to washings. These components do not need the observance of regulations or the usage of peculiar materials, but they must respond to the market, which aims to increase productivity. In order to do so it is necessary to employ faster and faster regulators and the ease to rapidly reconfigure the machine, if it changes the type of product in progress or its format.

The cylinders responding to the regulation ISO 15552 (diameters from 32 to 200mm) and ISO6432 (diameters from 8 to 25mm) can work high-speed, covering tens of thousands equivalent kilometres. The aim has been reached by employing a mixture of tribologic solutions and materials including: polyurethane gaskets, chromed and rebored rods with a very low superficial ridges, pistons control with PTFE slots or low-friction pistons in technopolymer; rod control with Teflon-coated steel bush; grease for other speeds.

In applications that require not only high speed but also a control of the motion and/or to change the work cycle, electrical cylinders are spreading. Metal Work, one of the first companies in the automation sector to believe in electrical cylinders, now offers a very wide range of solutions, which belong to the vast family called Elektro, with the same size of the pneumatic cylinders ISO15552, with diameters from 32 to 100 mm. It should be noted that the diameter is a fictitious data, since there is a screw with a ball recirculation leadnut inside the cylinder and the output force does not depend on the sleeve’s diameter but of the engine size and on the static and dynamic load of the electromechanical components. Then there are the rodless versions, where the motion can be implemented from screws with a ball recirculation leadnut for applications with high load and medium speeds, or belts with cogged pulleys in applications with low loads and high speeds.

All these components with electrical engines serve to freely modify the work cycles, to change the running by stopping in any position, to adapt the speed to the applied loads.

In the sector of the food industry there is a specific application of the Elektro series cylinders, where their typical performance are used at their best, and it is the one concerning the dispensers of liquid and semi-liquid food. Indeed, since the stroke of the rod can be controlled in a very precise way, by connecting the rod to a syringe it is possible to provide a specific amount of fluid. If you need to fill containers of a different volume, all you have to do is change the program of the cylinder’s stroke, dosing the quantity of food coming out of the syringe.

The electrical cylinders are more expensive than the pneumatic cylinders, even if the gap is decreasing.

Tailor-made components

The designers of machines that belong to the food industry often need special actuators, designed in stainless steel. Alfa Meccanica, a Metal Work society, is specialised in this field. A meeting between a customer designer and the Alfa Meccanica specialist often leads to the definition of a new product, which incorporates more functions and makes the solution easier.

Our product specialists support the customer in the study of the application and define the best product with him, be it standard or special manufacturing.

Such activity, which Metal Work has always undertaken on traditional pneumatic products, has built further momentum with the entry into production of electrical cylinders: in such case, the Metal Work specialist must have both mechanical and electronic skills.

It is a constant commitment, often ending with the identification of a complete solution to the application, rather than the identification of a product in a catalogue.

Metal Work is investing many energies in this regard and results are not far behind.

Corrado Tamiozzo

Responsible for internal office – Metal Work SpA

IMAGES

General Figure (no title) – food industry application

Compact cylinders INOX controlled

INOX fillings

Fillings F-E and F-NSF series

F series MRA BIT regulators

Air treatment groups series Syntesi anticorrosion.

Splash Area valves

Elektro cylinders